Lab on Disk Manual

Accessories

- BLED112 USB Dongle (https://www.eibtron.com/epages/eibtron.sf/en_GB/?ObjectPath=/Shops/eibtron/Products/807-7742)

Software

Labview 2013 or above

Drivers

- NI-VISA

- NI-488.2

- BLE Toolkit

Instructions: Getting the Mac Address and installing BLE toolkit

https://forums.ni.com/t5/Community-Documents/LabVIEW-BLE-Bluetooth-Low-Energy-toolkit/tac-p/3595025#M3652

- Get a BLED112 USB dongle (e.g. in Digi-Key)

- Install the drivers provided by Bluegiga (normally the dongle installs them by its own)
3. Verify the COM number for the dongle in Device Manager > Ports

- Download and install the LabVIEW BLE Toolkit

- Open the ble_scan_devices.vi example provided in
<Labview>\examples\BLE\Examples\ble_scan_devices.vi

- Select the COM port for your BLED112 dongle

- Run the VI

Instruction: Controlling with disk view

- Select “Disk View” in the mode control switch

- Select “Control disk” Tab

- Select the Com of the USB Dongle

- Put the MAC Address of the device (The MAC address (7C:EC:79:69:6D:AE) was used for testing with the prototype in lab. The address obtained from scan devices should be used)

- Select the Address Type (obtained from Scan devices. Normally: Public)

- Run the VI

- The Leds of state (Connect to device, Discover Handles and Write to device) should turn on

  - If they didn´t turn on, disconnect the dongle and reset the arduino. This mistake occurs when the communication is not closed correctly (Abort Execution is used instead of the Stop in the program).

- Control the Valves by turning ON and OFF the leds in the disk.

- Click Stop to end the communication (don’t use abort execution)

Instruction: Controlling Disk from File

- Select “From File” in the mode control switch

- Select Tab “Disk from file”

- Select File that want to be read.

  - Should be a .txt file.

  - The format is the next: 20,8000. The first 2 numbers correspond to the expander. The last numbers correspond to the state of the valves.

  - If time wants to be set between the instructions, the time should be entered with a t at the beginning: t,2000. This will create a waiting time of 2 seconds.

- Select the Com of the USB Dongle

- Put the MAC Address of the device (The MAC address (7C:EC:79:69:6D:AE) was used for testing with the prototype in lab. The address obtained from scan devices should be used.)

- Select the Address Type (obtained from Scan devices. Normally: Public)

- Run the VI

- Click Stop to end communication or the program will end once the instructions are over.

Example Instruction:

t1000 // waits 1 second

20,FFFF

23,FFF1

25,FFF3// the three instructions are executed at the same time

t4000 // waits 4 seconds

21,FFFF

27,FFF1

22,FFF3

// end of program